Camcorder and Zoom

- Use Laptop #1 (Zoom / Camcorder) and old Atem Mini
Connect micro HDMI to camcorder and larger HDMI to Atem Mini as usual

- Zoom settings
Audio…    Microphone: Black Magic Design   Speaker: Headphones
Video…     Black Magic Design

- Activate mic 1 and camcorder sound on Atem:  On / Off

- If using the Parish Hall Fender Amp, connected portable speakers, bluetooth mic...
Connect 15’ male to male 3.5mm audio cable to Fender amp’s phones output and Atem mic 1

- If not using Fender Amp use wifi mic located near the pulpit.
Use wifi mic receiver (to the left of soundboard on tech table). 
Attach 1/8” to ¼” adapter to 15’ audio cable.  Plug into wifi receiver and other end into Atem

- Connect black USB C (webcam out) to Atem.  Plug other end of USB A (laptop) into hub

Cables from left to right

- mic 1:  15’ braided 3.5mm male to male audio  (if using Fender amp)
or, mic 1:  wifi mic dongle (not pictured) (if using wifi mic)

- HDMI cable from camcorder (as usual)

- black USB C into Atem webcam output…   USB A into laptop.

- Power cable